2-Stage OpAMP Design (1st stage -> NMOS Diff Amp, 2nd stage -> PMOS CSA)

KARNATI PARANJAI

IEC2021097

Final year student, B-Tech ECE, IIIT Allahabad

Required Specifications of the Design:

| Low Frequency Gain (DC Gain) | 1000 (60dB) |
| --- | --- |
| Gain x Bandwidth product | 50 MHz |
| Phase Margin | 60 deg |
| Slew Rate | 30 V/usec |
| Load Capacitance | 1 pF |

| VDD | 1.8 V |
| --- | --- |
| ICMR(+) | 1.6 V |
| ICMR(-) | 0.8 V |
| Power | 400 uW |
| Technology | UMC-180 nm |

Design Parameters after Final Tuning:

| Channel Length (L) | 500 nm |
| --- | --- |
| Compensation Capacitance (Cc) | 325 fF |
| Tail Current (Iss) | 20 uA |
| M1, M2 | (w/l) = (3u/0.5u) |
| M3, M4 | (w/l) = (6u/0.5u) |
| M6 | (w/l) = (100u/0.5u) |
| M7 | (w/l) = (64u/1u) |
| M5, M8 | (w/l) = (8u/1u) |

Sample Outputs

- Vin = ICMR(-)

Schematic : (The DC operating point of each MOSFET is annotated beside)

Gain and Phase Plot:

Obtained:

| DC Gain = 67.5781 dB | Bandwidth = 53 – 54 MHz | Phase Margin = 57.5568 deg | Power = 346.698 uW |
| --- | --- | --- | --- |

- Vin = 1.2 V

Schematic: (The DC operating point of each MOSFET is annotated beside)

Gain and Phase Plot:

| DC Gain = 65.5928 dB | Bandwidth = 56-57 MHz | Phase Margin = 56.5794 deg | Power = 360.954 uW |
| --- | --- | --- | --- |

Obtained:

- Vin = ICMR(+)

Schematic: (The DC operating point of each MOSFET is annotated beside)

Gain and Phase Plot:

Obtained:

| DC Gain = 59.0958 dB | Bandwidth = 55-56 MHz | Phase Margin = 57.434 deg | Power = 371.412 uW |
| --- | --- | --- | --- |

Obtained Slew Rate and Power of the Design:

| Slew Rate | 61.5 V/usec |
| --- | --- |
| Average Power | 359.688 uW |

Process Corner Analysis:

Setting up the outputs and desired specifications are mentioned:

Results:

Corner analysis is setup for 3 processes typical, fast-fast and slow-slow, each at 3 different temperatures of -55oC, 27oC and 1250C.

Summary:

typical_0, fast-fast_0, slow-slow_0 -> corresponding process corner at -55oC.

typical_1, fast-fast_1, slow-slow_1 -> corresponding process corner at 27oC.

typical_2, fast-fast_2, slow-slow_2 -> corresponding process corner at 125oC.

Waveforms: